◉  PUBBETS WORKSHOP

Making Your 3D Puppet Models

A beginner’s guide: scan your real puppet → tidy it in Blender → export a .glb the builder can use

This guide walks you through making the very first model — the base "Classic Pubbet" — from start to finish. Once that base loads correctly in your builder, every other part (hair, glasses, outfits, shoes) is made the same way and sized to fit it. Take it one step at a time; you do not need any prior 3D experience.

The big idea You have the real, physical blank puppet. Instead of sculpting from scratch, you photograph it and let an app turn those photos into a 3D model (this is called "photogrammetry"). Then you clean that model up in Blender and export it as a .glb file. The builder already knows how to load .glb files.

# Part 1 — What you’ll need

- Your blank puppet (the base, no outfit) and a tape measure.

- A smartphone with a scanning app installed (see the table below).

- A computer with Blender installed — free from blender.org.

- Good, even lighting and a simple background. Daylight indoors is perfect.

## Pick a scanning app

Any of these work. If you’re unsure, start with Polycam — it’s the most beginner-friendly.

| App | Cost | Runs on | Best for |
| --- | --- | --- | --- |
| Polycam | Free tier / paid | iPhone, Android | Easiest, fast results |
| KIRI Engine | Free tier / paid | iPhone, Android | Good free exports |
| RealityScan | Free (Epic) | iPhone, Android | High detail, free |

# Part 2 — Scan the puppet

The goal is a clean set of photos all the way around the puppet. Aim for 40–60 photos.

- Stand the puppet up on a table, ideally on a plain surface. Put something small and patterned under it (a sheet of newspaper works) — it helps the app track position.

- Fill the frame with the puppet, not the room. Get close enough that it takes up most of the screen.

- Circle it slowly at a low angle, taking a photo (or letting the app auto-capture) every 10–15 degrees. Do a full lap.

- Do three laps at three heights: one low looking slightly up, one at the puppet’s middle, one higher looking slightly down. This captures the top of the head and under the chin.

- Keep the puppet still — move yourself around it, don’t rotate the puppet, or the background won’t line up.

- Process the scan in the app, then export as .obj or .glb (either is fine — Blender opens both).

Measure while you’re at it Before you put the puppet away, measure its real height, width, and head diameter with your tape measure and jot them down. You’ll use these in Blender (Part 4) so the digital puppet is the correct real-world size — which keeps every future part perfectly in proportion.

# Part 3 — Get comfortable in Blender

Blender looks busy at first, but you only need a handful of controls. Open Blender, and when the startup box appears, click anywhere to dismiss it. Delete the default cube by clicking it and pressing X → Delete.

## The three moves you’ll use constantly

- Rotate the view: hold the middle mouse button and drag (or two-finger drag on a trackpad).

- Pan: hold Shift + middle mouse and drag.

- Zoom: scroll the mouse wheel.

Import your scan with File → Import, and choose glTF (.glb) or Wavefront (.obj) to match what you exported. Your scanned puppet appears in the scene.

# Part 4 — Clean up and size the model

Raw scans are messy — there’s usually a chunk of the table, stray blobs, and far more detail than a web app needs. Here’s the tidy-up:

- Delete the floor/background: press Tab to enter Edit Mode, box-select the unwanted bits (press B then drag), and press X → Vertices. Press Tab again to leave Edit Mode.

- Center it: Object → Set Origin → Origin to Geometry, then press Shift+C to bring it to the center.

- Set the real size: press N to open the side panel, and under Dimensions type your measured height (set units to your preference in Scene Properties). Now it’s true-to-life.

- Reduce heaviness: with the puppet selected, add a Decimate modifier (wrench icon → Add Modifier → Generate → Decimate) and lower the ratio until it looks smooth but simpler. Aim for a model that isn’t sluggish to spin.

- Face forward: rotate the puppet so it faces –Y (front view is Numpad 1). The builder expects parts to face the camera.

Don’t aim for perfect For the first pass, "recognisable and clean" beats "flawless". You can always re-scan or refine later. The priority is getting one real model loading in your builder so you can see the whole pipeline work end to end.

# Part 5 — Color and texture (optional for v1)

Scans often bring their own color ("texture") along. If yours looks right, skip ahead. If you want to set a flat color instead:

- Open the Material Properties tab (the checkered sphere icon on the right).

- Click New, then click the Base Color swatch and pick a color.

For the base puppet, remember the builder can recolor it to your 9 body colors automatically — so a neutral light-grey base actually works best, letting the color options do the work.

# Part 6 — Export the .glb

- Click your puppet to select it.

- Go to File → Export → glTF 2.0 (.glb/.gltf).

- In the export panel on the right: set Format to glTF Binary (.glb), and tick Selected Objects so only the puppet exports.

- Name it clearly — e.g. classic-pubbet.glb — and save it into your app’s assets/models/ folder.

# Part 7 — Plug it into the builder

- Open admin.html in your builder.

- Find the Puppet category → the "Classic Pubbet" row.

- In its Model file box, type the exact filename: classic-pubbet.glb.

- Click Save options.json, replace the old data/options.json with the downloaded file, and reload the builder.

Your real puppet now loads in 3D. 🎉 If it looks too big, too small, or rotated, that’s normal — the builder auto-scales, and you can nudge orientation back in Blender and re-export.

# Suggested order to build your models

Work outward from the base so everything fits together:

- 1. Base "Classic Pubbet" — the foundation everything else sits on.

- 2. Eyes and nose — small, quick wins that add personality.

- 3. Hair styles — model onto the head; the colors are just material changes.

- 4. Outfits — t-shirts, dresses, premium looks.

- 5. Glasses, shoes, accessories — the finishing touches.

A few golden rules for models that behave in the app

- Keep each part a separate .glb (one for hair, one for glasses, etc.).

- Center each part at the origin (0,0,0) and face it forward (–Y).

- Keep polygon counts modest — these spin in a web browser and on phones.

- Apply materials/textures before exporting, and tick "Selected Objects".

- Host finished .glb files in Shopify Files (paste the CDN URL into admin), or keep them in assets/models/ while testing.

Pubbets Workshop · handmade to order